
Equipment Operation Sheet:  How To Replace a Lamp on an LCD Projector

Type:  Hitachi model CP-RX60 Multimedia LCD Projector

[image: image1.jpg]



You Will Need the Following Items:
1 Phillips head and 1 flat head screwdriver







1 Replacement lamp

Instructions

1. Be sure the unit is unplugged and the lamp has had sufficient time to completely cool down, about 1 hour to be safe.

2. Carefully flip the unit over so that the bottom is exposed.

3. Find the area on the bottom of unit that has a backing to be removed by one silver screw.  The lamp cover looks like this:

[image: image2.jpg]



4. Unscrew the screw with a Phillips head screwdriver to reveal the lamp casing.  The unit will look like this after you have removed the cover for the lamp and its case:

[image: image3.jpg]



5. Now, remove the two silver screws (as shown above) with a small flathead screwdriver.

6. Lift out the case with the lamp attached.  The lamp looks like this:

[image: image4.jpg]



7. Carefully insert the new bulb in its case.  Secure with the two silver screws.

8. Replace the cover with the tabs aligned properly, then tighten with screw.

9. Carefully turn the projector right side up and turn on the unit to ensure the lamp is working properly.

If you have any other questions concerning the installation of a bulb for this unit, please click on the link below for the owner’s manual for this model.  Refer to the table of contents for step-by-step instructions:

http://www.aboutprojectors.com/pdf/hitachi-cp-rx60-manual.pdf






Four Squares
LCD Projector Bulb Replacement
1

