
Equipment Operation Sheet:  How To Replace a Bulb on an Overhead Projector

Type:  Apollo Eclipse

[image: image1.jpg]



You Will Need the Following Item:
1 replacement bulb

Instructions

1. Be sure the unit is unplugged and the lamp has had sufficient time to completely cool down, about 1 hour.

2. Press the black cover release button on the side of the machine while lifting the flatbed cover at the same time to reveal the inside of the machine.  The release button is on the right side of the overhead projector and looks like this:

[image: image2.jpg]



3. This unit has two bulbs, “1” and “2”.  For the bulb that needs replacing, push the release lever back towards you in order to unseat the bulb:

[image: image3.jpg]



4. Carefully lift out the bulb using extreme caution as not to break it.

5. Push the release lever forward in order to install the new bulb.

6. Place the new ENX bulb, inserting with care as not to break it.

7. Close the lid until you hear it “click” shut.

8. Test the new bulb.  Be sure to select the proper bulb position, “1” or “2” by sliding the selection mechanism.







Four Squares
Overhead Projector Bulb Replacement
1

